Gás Detector Project

Introduction:

This is a gas detecting circuit capable of sensing many different types of gases.

The sensor used is the GH-312 and from the datasheet it is capable of sensing gases like smoke, liquefied gas, butane and propane, Methane, alcohol, hydrogen, etc.

Schematic:

Part List:

R1		1K resistor

R2		1K resistor

P1		100K potentiometer

C1		10uF cap

C2		100nF cap

C3		100nF cap

C4		15pF cap

C5		15pF cap

Xtal		8Mhz crystal

Led1		3mm red led

Piezo		Piezo

LCD		8X2 LCD

IC1		16F84A microcontroller

VR1		7805 regulator

GH-312	Gas Sensor

PCB:

The PCB used for this Project is single layer and its size is 65.95mm x 59.25mm.

Top Side

Here is the top view with the components

Bottom Side

Here’s the bottom side.

I’ve used a 9V battery for this project because the sensor requires a 9V input.

The output when the sensor is triggered is always 5V which is perfect for a direct connection to any microcontroller.

Although I’ve used a 9V battery, any 12V power supply ( a small wall transformer for example ) will work perfectly since the sensor can handle from 9V up to 20V and the microcontroller’s voltage is regulated by a 7805 regulator.

Testing:

The first tests were made with the circuit mounted on a breadboard.

After initialization the circuit will enter a normal state where it detects no gas.

The display shows "Sensing...No Gas !”.

To test the sensor I used my portable gas soldering iron with the gas coming out pointed to the sensor.

The sensor is able to detect the gas and the microcontroller will trigger a flashing led warning and sound.

The sound is produced by a small piezo and the display show the message "Found Gas".

When the air is clean again and the sensor does not sense any gas, the circuit will return to it's normal state turning off both led and piezo sound.

Conclusion:

It's a pretty cheap and easy to assemble circuit.

Does not require too many parts and the microcontroller is very easy to find ( the famous 16F84A from microchip ).

Since it’s used a small lcd ( 8x2 ) this project can be portable.

Also this sensor senses several types of gas and it’s pretty stable.